| jtoolman Newbie Offline Posts: 12 | Re: Refillable catridges on the 3800 - question. « Reply #3 on: March 02, 2014, 09:03:02 PM »; Reply Quote. Actually there is not need to drill out the rear cap of the valve body which contains a little round piece of plastic to allow ink out but not in. There is NO need for this little body which holds that little disk in. It's only function is to prevent you from injecting ink in. After removing the front retaining ring, rubber seal, piston, and spring, you insert a blunt needle and gently push until the rear cap just pops off internally. It is only held by two little nipples. It will now simply float inside the bag NOT disturbing the flow of ink into the printer when printing. The PRO 3800 and 3880 have anti backflow lines anyway. Pressurized ink can enter but cannot black back into the carts. Just so everyone knows. I have documented this whole process in 5 videos in youtube. Just search for "Want to refill and reset your OEM PRO 3800 / 3880 Carts? " … Here is my new video on the Easy method of modifying the carts. https://www.youtube.com/watch?v=7kCc5fwssVM |
| --- | --- |

http://www.luminous-landscape.com/forum/index.php?topic=34957.0;wap2

arcman:

…

There is an extremely small (almost invisible) plastic disk in the back of the neck of the inner bag that works as a one way valve.